# Indice delle lezioni

1. Sensori: Principi di funzionamento, caratteristiche comuni a tutti i sensori e trasduttori. Esempi di sensori: Velocità, Posizione, Pressione, Temperatura, Sensori di radiazione, di luce, Fotodiodi, CCD.

2. Sistemi e modelli matematici – nozioni di base sui controlli automatici

3. Condizionamento dei segnali di misura: necessità, amplificatori operazionali: varie configurazioni, Filtri.

4. Digitalizzazione delle informazioni: DAC, ADC, e loro caratteristiche principali, esempi di tecniche di conversione. Teorema del Campionamento, alias, effetto del sovracampionamento e decimazione

5. Trasmissione delle informazioni: Generalità, varie topologie, trasmissione seriale, trasmissione parallela, sincrona/asincrona. Reti e protocolli di rete TCP/UDP IP. Caratteristiche dei bus: linee dati, linee indirizzi, linee di controllo. Arbitraggio

6. (optional) Sistemi operativi e S.O. in real time. Microcontrollori vs microcomputer, linee di I/O analogiche e digitali

# General characteristics of sensors

The first step for a data acquisition system is the sensor. Sensors are devices capable of transforming the physical quantity to measure into a signal that can be elaborated. A sensor is composed by an input interface, a transducer and an output interface.

- Figure 1-1: An example: a cosmic radiation sensor with scintillator and a photomultiplier

The transducer – the main component of the sensor – is a device that transforms a form of energy in an electric signal. An example  is the thermoresistance; it changes its resistance as a function of the temperature, thus it is a  sensor that translates a change in temperature into a change of electric resistance that can be sensed as a change of potential at its extremes when a current is supplied.

As one can imagine, often the distinction among transducer and sensor is not net, and, in the everyday practices they are mostly synonyms.

## Physical quantities and signals

Physical quantities that can be sensed by a sensor can be classified in:

- Continuous quantities: can assume continuous values within a certain interval, i.e. the measurements taken in two infinitely near points can assume any of the values within the measurement interval) .

- Discrete quantities:  the result of a measurement can assume only a set of discrete values, e.g. the sense of rotation can be +1 or -1, or the number of particles emitted can be only an integer.

The information linked to continuous quantities are said to be analogic signals, while those pertaining to discrete quantities are associated with logic signals – for two possible values only – or codified signals for more than two values admissible.

An analogic signal is continuous in time and can assume a continuous set of values. The acquisition system, take some time to elaborate the signal in input, so it will not be capable of following the evolution of the signal in every moment but it will sample it at different discrete time intervals. Also, the precision of the elaborators, does not permit to distinguish two infinitely near measurements, so, an approximation is needed to the nearest value allowed by the resolution of the sampler. This operation is the quantization of the sampled signal. The quantized information taken at discrete time intervals is the digital signal. The digitization and the properties of sampled signals are the object of lesson xxx

- Figure 1-1-2: Sampling a continuous signal at regular intervals. The dots represent the nearest to true values permitted by the sampler resolution.

## General functional characteristics of sensors

### Static characteristics

- Figure 1-1-3: A sensor with N inputs and M outputs

Sensor

X1

X2

XN

YM

Y1

If, like in systems theory, we represent a sensor as a “black box” with one or more inputs and one or more outputs, we can establish a input-output relationship that is known as the sensor’s characteristics. This quantity, when calculated for input signals that do not change in time, or change slowly enough to be considered not influent on the sensor response, is a pure algebraic relationship that is called static characteristics.

Calling the nominal inputs/outputs with the capital X and Y, and the actual values with the lowercase letters, we can write the static model of the sensor as the set of M equations: yi=fx1…xN

Then, with respect to the Nominal values we can write: Δyi=yi- Yi =yi -fX1…XN,  and, linearizing:

Δyi=j=1NaijΔxj;         aij=dyidxj

i.e. an algebraic expression. A typical example of such a linearization is the static hybrid parameters model of a transistor, as a 2x2 system.

### Dynamic characteristics

- Figure 1-1-4: RC Series schematics

When the signal varies in time with frequencies high enough to affect the sensor’s response, it is necessary not only the equations linking input and output variables, but also the link of in/out time derivatives. This accounts for the dynamic characteristics of the sensor, that now is a set of differential equations.

For example, if the sensor behaves as an RC-series, we must write for the voltage Vo across the capacitor:

Iint=CdVotdtVin=RIin+Vo=RCdVotdt+Vo

I.e.

dVotdt=-VotRC+VitRC

x=Vo;y=x;u=Vi
⇒xy=ABCDxu=-1/RC1/RC10xu

Where we expressed the system of equations linking input, output and their derivatives in matrix form. This will be clear in the part dealing with systems theory, just note that in this representation, it is a set of linear equations not depending on the time, i.e. a Linear Time -Invariant system (LTI).

### Linearity

A system is linear if its characteristics Y=f(X) satisfies the distributive property w.r.t. the sum:

fc1x1+c2x2=c1fx1+c2fx2;∀c1,c2∈R

For a linear transducer, the linearity is expressed as Y=kX, where  the constant k is the linear gain.

With this definition, an offset breaks the linearity, in fact: if Y=f(X)=kX+d; then f(X1+X2)=K(X1+X2)+d that is different from the sum of the two measurements Y(X1)+Y(X2)=kX1+d+kX2+d=k(X1+X2)+2d.

We can define a linearity error as the maximum deviation from the best linear fitted characteristics. In percentage, the non-linearity error:

ϵNL%=100⋅ΔYYM-Ym

Where the difference at the denominator is the measurement range.

The gain error is the difference between the gain of the fitted line w.r.t. the ideal transducer linearity gain.

### Resolution, precision and accuracy

Resolution is the maximum input variation that  does not give rise to an output change, i.e. the minimum possible read-out value.

Precision is a measure of the repeatability of the measurement. It can be quantified as the spread of the repeated measurements around a mean value:

ϵP=100⋅EMaxYM-Ym

Where  EMax is the maximum spread (or, more often EMax=3σ).

Accuracy is the a measure of the deviation of the sensor output from the true value of the input, i.e.  the value an ideal sensor would give.

ϵA=100⋅ΔYideal

- Figure 1-1-5: Precision and accuracy

Accurate not precise

Precise and accurate

unprecise and inaccurate

Precise not accurate

Sensitivity is the maximum sensor’s output variation to input maximum variation ratio:

S=YM-YmXM-Xm

All these quantities, can be functions of the frequency if the sensor characteristics depend on it.

## Some classification of sensors

We can classify sensors in categories, following some of their main physical characteristics:

- Continuous output sensors: all the information is acquired, e.g. a photodiode

- Logic output sensors: only part of the information is acquired, e.g. a threshold detector

- Modulating sensors: need a service source of energy and the object of the measurement modulates a field, e.g. a laser speed-meter

- Self-exciting sensors: do not need a source of energy, e.g. a thermometer.

# Physics of sensors

Sensors use several physics properties to accomplish their tasks. Here we try to give a short list with examples:

## Resistive sensors

Here, we mostly use the relationship between the resistance R , the length L and the section S of the conductor through the resistivity:

R=ρLS

If a difference of length is sensed the resistance is changed accordingly, realizing a position or a length sensor.

If is the section to change, the variation of resistance can realize a strain-meter or a deformation meter.

The resistivity is a function of the temperature and in certain cases of the illumination. For example, if we consider the conductivity (the inverse of resistivity) as a function of the carrier densities n and p  and their mobility:

σ=eμen+μhp

It increases with temperature, as the density of carriers in conduction band incr witheases the Maxwell-Boltzmann distribution:

n=2kT2πℏ232memh34e-Eg2kT

Where Eg is the energy gap between valence and  conduction band. The resistance decreases accordingly, so it is possible to realize a  thermoresistance, i.e. a temperature sensor.

With the same equations is clear that if we illuminate the semiconductor with light of frequency large enough to promote carriers in conduction band: hν≥Eg, we realize an illumination sensor.

## Inductive sensors

# 3. Acquisition and control chain

Sensor

Signal conditioning

Signal digitize (ADC)

On-board elaboration

DAC

Control signal

analog signal

digital signal

analog signal

Signal conditioning

Actuator

analog signal

Storage/ext elab.

Transmit data

digital signal

Threshold and combining

Control signal (logic)

analog signal

Raw data

3.1	Signal conditioning

### 3.1.1	Necessity of conditioning

To acquire the signal from sensors, it is often necessary to amplify, filter, adapt it before passing it to the digitizer or other instrumentation - e.g. a scope. These operation are collectively called signal conditioning. To make a practical example, if the sensor exit is a continuous voltage between 0 an 10 V and the digitizer input accepts a voltage from -3V to 3V, to use the whole digitizer scale, it is necessary to adapt the sensor’s signal to the ADC input: i.e.

- Add an offset of -5V, to change the scale 0 to -5

- Reduce the [-5 , 5]V range to [-3, 3]V

Another typical case, is when the sensor’s output id very feeble, e.g. for a thermocouple. In this case, the signal must be amplified. This operation must be performed as much as possible near the signal source, so to amplify the signal without amplifying the noise picked-up during the transport from the sensor to the digitizer.

A

B

- Figure 0-1: A II order filter with Op-Amp

Let’s calculate the TF of a II order filter as in figure, where the G’s are the complex conductances Gi=1/Zi:

In A:

Thus:

G1VA-Vin=G2VB-VA+G3V0-VA

VAG1+G2+G3-G1Vin=G2VB+G3V0

In B:

VB=V0G4VB=G2VA-VB
⇒VA-V0=G4G2V0

So

VA=G4+G2G2V0

And, substituting:

G1VA-Vin=-G4G2V0G2+G3
VinG1=G1G2G2+G4+G4G2G2+G3V0
Trf=V0Vin=G1G2G1G2+G4+G4G2+G3

By choosing the couple of conductances it is possible to realize a low pass, high pass or a resonant filter.

Now, for a low pass filter:

G1=1/R1;G2=1/R2
G3=sC1;G4=sC2

Trf=1s2C1C2R1R2+sR1+R2C2+1=1s2+2as+a2+b2

Where a and b are respectively the real and imaginary part of the poles

p1,2=a ±i b

For a Butterworth, the poles are at ±π4 in the negative real part of  unit circle, i.e.: p1,2=22±22i

thus:

C1C2R1R2=1
R1+R2C2=2a

By calling R1C1R2C2=1/ωc2 and R1+R2C2=2ξ/ωc with  ξ=R1+R2/R2 , then, we obtain:

Trf=ωc2s2+2ξωcs+ωc2

The third equation comes from the choice of the cut frequency:

R1C1R2C2=ωc-2=12πfc2

By putting R1=R2=1 then, C1C2=1 i.e. C1=1/C2 and C2=a=22⇒C1=2

If  s=iω, at ω=ωc,  Trf=ωc2/-ωc2+2ξ ωc2+ωc2=1/2iξ, so at the cut frequency, the output is ½ the input and the phase ϕ=π2

To find the components, we can choose one parameter arbitrarily and find the others, e.g., if I want fc=1kHz,  if we choose R1=R2=10kΩ:

R1R2=108
C1C2=1C1C2R1R2=12πfc2
C1C2⋅108=12πfc2⇒12π2=C1C2⋅1014
C1'=C1⋅10-72π=1.414/2π⋅10-7=22.5nF

C2=1/C1⋅10-7/2π=11.3nF

- Figure 0-2: pspice simulation of the 2nd order Butterworth filter and its transfer function vs frequency

To build a 4th order filter, one can cascade two 2nd orders as in figure below .

- Figure 0-3: Top transfer function vs frequency, bottom schematics of the 4th order Butterworth filter

The transfer function is the product of the two. To be a Butterwoth, all poles must be  equispaced on the unit circle and with real part negative, so the angles are π8 apart. We obtain:

Poles (a ± ib) 	Quadratic Expression

-0.9239 ± i0.3827	 s² + 1.8478s + 1

-0.3827 ± i0.9239 	s² + 0.7654s + 1

If we fix the resistors to be 1, we remain with the conditions:

C1C2=1=ωc-2
2C2=2a

So C2=0.9239  and C1=1.082, C4=0.3827, C3=2.6113; this sets the capacitors-to-resistors rate.

So, ideally, if we set the resistors to R=1kΩ we get C= 1mF, but the cut frequency remains 1Hz. If we want it to be 1 kHz, we still must reduce the capacitor by a factor 2π⋅1000, we obtain

C2=147nF

C1=172nF

C4=61nF

C3=416nF

## Spectral behaviour of sampled signals

Let’s denote the continuous signals as functions, f(t), of the continuous time while the sampled signal is a function F(n) of the discrete time represented by the sample number n. Let us begin to examine the differences between the continuous and the discrete version of the complex exponential function. For the continuous ft=eiωt; the larger is ω the larger is the rate of oscillations and is periodic for any value of ω.  The discrete counterpart has a different behaviour, in fact, considering the complex exponential with frequency Ω0+2π :

Fn=eiΩ0+2πn=eiΩ0ne2iπn=eiΩ0n.

As we see it has the same frequency as the function with frequency Ω0, the same being for signals with frequency Ω±2π, Ω±4π. Therefore, in considering discrete time complex exponentials we need to consider only an interval of length 2π.

As far as the periodicity is concerned, in order for the signal to be periodic of period N, it must satisfy the relation:

Fn=Fn+N⇒eiΩn=eiΩn+N

or equivalently: eiΩN=1, that is ΩN = 2kπ, i.e. eiΩNis not periodic for any Ω, but only for Ω=2πk/N

The discrete-time sequence obtained by sampling a continuous complex exponential signal at equally spaced points in time is:

Xn=eiωnT

that is a discrete time exponential of period ωT, being T the interval between the samples, i.e. the reciprocal of the sampling pulsation: thus, if ωs=2π/T is the sampling frequency, the function will be periodical only if ω/ωs is a rational number.

### 3.2.1	The sampling Theorem

Any continuous signal x(t), sampled with a period T can be represented in terms of a sum:

xpt=n=-∞∞xnTδt-nT

By Fourier transforming this expression, we see that it is the convolution:

Xpω=12πXω*Pω

where

Pω=2πTk=-∞∞δω-kωs

so

Xpω=1Tk=-∞∞Xω-kωs

That is Xpω is a sum of replicas of Xω scaled of 1/T

If the spectrum of the original signal has a maximum frequency ωM, then if ωM<ωs-ωM i.e. $ωs>2ωM there is no overlap of the replicas and the signal can be recovered exactly from the sampled signal by applying a low-pass filter with cut frequency ωM<ωc<ωs -ωM.  This is what is called the sampling theorem, the frequency ωs=2ωM is called the Nyquist frequency.

The sampling theorem can be enunciated as follows:

Given a band-limited signal x(t) with Xω=0 for each ω>ωM, then x(t) is uniquely determined by its samples xpnT if

ωs>2ωM

where ωs=2π/T is the sampling frequency.

### 3.2.2	Numerical filters

If the representation of our signal is a series of samples x(n) sampled at intervals of T, its derivative, at first order is given by:

dxtdt=xnT-x[n-1]TT

We could improve the precision, by using also the forward derivative

dxtdt=x[n+1]T-xTT

And using Taylor series for both :

xn+h=xn+hdxdt+h212d2xdt2+…
xn-h=xn-hdxdt+h212d2xdt2+…
so the difference, gives

xn+h-xn-h2h=dxdt+16Oh2
Now, restricting to the normal incremental ratio, the second derivative is

d2xtdt2=1Txn+1T-xnTT-xnT-xn-1TT=
=1T2xn+1T-2xnT+xn-1T

In general the k-th derivative is:

dkxtdtk=m=0Mbmx[k-m]

Thus, applying the z transform: Zdkxtdtk=m=0Mbmzm-k=m=0MbmXzz-k

This allow us to write differential equations for discrete systems as difference equations. It means, that if the equation for a filter is:

k=0Nbkdky(t)dtk=m=0Mamdmx(t)dtm

We can write, for a discrete filter:

k=0Nakyn-k=m=0Mbmxn-m⇒yn=1b0m=0Mamxn-m-k=1Nbkyn-k

That is a recursive equation, since we must know all the n-N values of the exit to get the present value.

If N=0, the difference equation is non recursive and its response to a step is finite, defining the so called Finite Impulse Response (FIR) filter, the other case being infinite response (IIR).

In the most general case of the IIR, by applying the z transform:

Yz=l=0Lalz-lXz+m=1Mbmz-mYz ⇒
m=0Mbmz-mYz=l=0Lalz-lXz

So that we can write the transfer function:

Y(z)=H(z)X(z)⇒H(z)=Y(z)X(z)=m=0Mbmz-mn=0Nanz-n

A difference between the continuous and discrete time transfer function, is that in the latter, also a delay has a rational TRF; in fact, if y[k]= x[k-h], then Yz=z-hXz so that H(z)= z-h.

As for the continuous case the poles of H must have real part negative for the response to be finite.

A typical example is the moving average filter (is a low pass filter). Its time definition is that the n^th exit is the average over the k preceding inputs:

yn=1K+1m=0Kxn-m

Yz=XzK+1m=0Kz-m =XzK+11-z-1K1-z-1⇒
Hz=1K+1zK+1-1z-1z-K

This transfer function has K poles in the origin, and K zeros equispaced on the unit circle, representing the blocked frequencies. If one takes z=eiω:

1N+1zN+1-1z-1=1N+1eiωN+1-1eiω-1=1N+1eiωN+12-e-iωN+12eiω2-e-iω2eiωN2≈sinN+12ωN+1sinω2

- Figure 0-4: the transfer function of the MA filter is a sync.

Now let’s find the digital equivalent of a RC filter.

- Figure 0-5:RC circuit

The differential equation is:

V=RCdVcdt+Vc

or, taking the complex impedances:  Vc=V/sCR+1/sC=V1RCs+1 . By calling RC=τ=1/ωC, Y=X1τs+1, i.e.

Hs=1τs+1

Transforming the differential equation in a difference equation: x[n]=RC/T(y[n]-y[n-1])+y[n], i.e., yn1+τ/T-yn-1τ/T=xn. By z-transforming, one obtains: 1+τ/T-z-1τ/TYz=Xz, so:

Hz=11+τT-τTz-1=ωs/ωcωs/ωc+1-z-1=ββ+1-z-1=b1-az-1

Where b=1ωs/ωc+1 and a=ωs/ωcωs/ωc+1.

So, given the values in figure: R=1k; C=1nF, ωc=1MHz, if ωs=10MHz, b=1/11; a=10/11, so the difference equation is: y[n]=0.09x[n]+0.9y[n-1]